Fossil Fuel and Heat Pumps

using

A/TCONT80 series or Honeywell TH8300 series

There has been misunderstanding and confusion about the capabilities of the Honeywell TH8300 series Vision Pro thermostats and the American Standard and Trane adaptations of them.  Contrary to popular opinion, these thermostats do NOT eliminate the desirability of the TAYPLUS103 fossil fuel kit!

Simply put, the TAYPLUS103 will allow the furnace to take over for the heat pump whenever second stage of the thermostat calls, no matter what the outdoor temperature.  The thermostat alone will not do this!  At any temperature above the outdoor thermostat set point, the thermostat will lock out the furnace and only allow the heat pump to run no matter if second stage is calling.

For example, if a night setback is used, above the outdoor thermostat setting, the thermostat alone will recover with heat pump only.  With the TAYPLUS103, the furnace will be brought on to rapidly warm the house.

Further, the thermostat outdoor temperature selection option only goes down to 15F.  The TAYSTAT250 adds the flexibility of being adjustable over a lower range.

These thermostats continue to be a very versatile and cost effective control and we still recommend them. For the present, we must recommend that even if one of the above thermostats are used with a heat pump install on a gas or oil furnace, the TAYPLUS103 must be installed.